# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Procurement of fire protection uniforms, equipment/tools and fire engine (light) with high pressure pump for project FIREGUARD

# p 1 /…

Publication reference: RORS00061–PP2–TD03.2

LOT no.1 - Personal protective uniforms for firefighters

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

General Requirements

- All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

- The tenderer is required to provide the specifications of the offered items in the Technical Offer, including details such as the manufacturer, product type, model, and country of origin. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Protective uniform, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: The suit (jacket and trousers) made of fireproof fabric is in the base of the suit. Single-layer firefighter suit, suitable for outdoor firefighting and technical interventions according to EN 15384. Material: 50% Teijinconex / 49% Lenzing FR / 1% antistatic - 230 g/m2 Material color: navy blue JACKET: - quality robust main zipper with closure -collar with Velcro closure -yellow perforated reflective strips, 5 cm wide, double stitched -4 textile loops in the chest area -2 loops with D-rings - one under the radio pocket, the other in the lower left pocket -1 radio pocket on the left side, large pocket on the right side -2 large lower inner pockets arranged vertically -pockets with Velcro flaps and reflective strip holders for better grip and additional visibility -flashlight holder with Velcro adjustment on the main zipper flap -sleeve ends with Velcro adjustment and elastic band for better fit -most stressed seams secured with reinforced stitching -extended back of the jacket for safe overlapping with trousers -large hanging loop on the inside of the collar • "VATROGASCI DVD" two-line inscription on the back in the standard version TROUSERS: -Knee reinforcements made of Kevlar with silicone-carbon coating -Yellow perforated reflective tapes, 5 cm wide • Elastic waistband for better comfort -2 pockets in the waist area with zipper closure -2 large cargo pockets with Velcro flaps and practical holders made of reflective tape for easy opening -Raised rear of the trousers for safe overlapping with the jacket -Width adjustment at the ends of the legs with Velcro -Zippered vent and additional fastening with a press stud -1 D-ring in the right cargo pocket • The most stressed seams are secured with reinforced stitching -Large loop for hanging on the back |  |  |  |
| 2 | Protective footwear, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: Half-boots are made from natural box leather,hydrophobized, with textile (Cordura) and rubber/ polyurethane sole. The footwear is entirely made in black color. The upper (the vamp, the collar, and the upper part) is made of hydrophobized cowhide leather, with a thickness of 2.0 ± 0.2 mm. The collar is made of natural napa leather. The upper and lower parts of the tongue are made of textile material (Cordura) in black color. The vamp is reinforced in the front part (forefoot) with an additional layer of natural leather coated with a thin layer of PU (Polyurethane), resistant to wear and bending. The lining is made from a four-layer textile laminate material with a climate membrane, ensuring waterproofness from the external environment and moisture vapor removal from the inside of the boot. It is resistant to water, wear, color fading, water vapor, and heat resistance. The tongue is jointed and part of the upper (gusseted tongue), fully protecting the instep. It prevents water, snow, and mud penetration. The tongue height meets qualitative and aesthetic conditions, ensuring that after fastening the footwear, its top edge is not lower than the height of the collar, regardless of the user’s ankle circumference. The footwear is equipped with a perforated collar filled with sponge-like material on the collar of the boot. The shoes are fastened securely and quickly with reinforced laces passing through metallic fittings, which are fastened only over the material surface and equipped with an inner washer to prevent tearing. One boot is fitted with a total of 4 pairs of metal eyelets and 3 pairs of metal stoppers. On the back part of the footwear, between the upper and the collar, there is a soft material that ensures free movement for the foot, enabling easier walking during stepping or running. The sole is made of Rubber/Polyurethane/PVC, with increased height at the front and rear parts, resistant to slipping, liquid fuels, with a shock absorber in the heel, resistant to bending, cracking, and wear. The heel is reinforced with a PVC stabilizer. The midsole is equipped with a "Fast Rope" system for quick climbing with a rope. The insole is anatomically shaped, covered with a three-layer textile material, and is removable. The declared sizes range from 36 to 48, in accordance with the SRPS ISO 9407 standard. Size intervals are 0.6 mm and 0.7 mm. Sole bending resistance SRPS EN ISO 20344:2022, section 8.6 max. 4 mm Abrasion resistance SRPS EN ISO 20344:2022, section 6.12: Water vapor permeability ASTM E96/E96M:2016 >1000 g/m²/24h Heel slip forward according to SRPS EN ISO 13287:2020 ≥0.13 Heel slip backward according to SRPS EN ISO 13287:2020 ≥0.18 SRPS EN ISO 20347:2022 – Personal protective equipment – Occupational footwear The supplier must provide: - Certificate of type examination - Report on testing of basic and auxiliary materials - Declaration of conformity Manufacturer’s declaration with instructions for use and maintenance |  |  |  |
| 3 | Fire helmet, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: The helmet is made of materials that are resistant to mechanical shocks and punctures, heat in accordance with the standards EN 16471:2014; EN 16473:2014; EN 12492:2012; EN 1385:2012. The helmet is designed on the basis of a hard shell, is of universal size, with adjustment using a dial from 47 to 68 cm, has ventilation openings and is equipped with chin straps and a shock-absorbing insert. The helmet should have a short neck protector made of non-flammable material. It should have eye protection glasses and have a lamp holder compatible with lamps (PELI TORRANCE CA, USA II2G EX eibIIcT4). It should also ensure the unhindered use of a half mask. Helmet color - yellow with reflective gray stripes. The weight of the helmet without additional equipment must not exceed 1000 grams. |  |  |  |
| 4 | Firefighter belt, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: The firefighter's belt, as part of the firefighter's personal protective equipment, serves to enable uninterrupted work, rescue and self-rescue at heights (depths). The firefighter's belt is supplied with a carabiner secured to it with adequate holders. The type A safety belt.The belt must be manufactured in accordance with SRPS Z.B1.050. |  |  |  |
| 5 | Protective gloves, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: Protective tactical gloves made of non-combustible material should provide mechanical protection of the hands and at the same time enable unhindered and comfortable work of firefighters and rescuers during interventions on fires in open areas (contact with incandescent materials of class A, unhindered work with equipment such as a chainsaw, backpack, etc.). Leather gloves for firefighters that meet the standards EN 659 + A1:2008 + AC:2009; EN 21420:2020; 100% Kevlar knitted wrist cuff. |  |  |  |
| 6 | Protective half masks with filter, Quantity: 21 Units |  |  |  |
|  | Required technical characteristics: Protective half mask with P3 filter for respiratory protection according to EN 140:1998. The half mask made of the latest generation TPU composite offers lightness, flexibility and comfort for a respiratory protection solution. The hypoallergenic TPE material does not cause allergic reactions, even after prolonged use, and does not irritate the skin; it is comfortable, odorless, very hygienic and suitable for many processes. In the main body there is a filter connector and an exhalation valve, consisting of a valve holder and a membrane, which facilitates the escape of exhaled air. The half mask is equipped with a comfortable neck and head strap that can be easily adjusted to the right length. The mask should be designed in such a way that it can be used without hindrance with a light helmet and safety glasses. Weight: maximum 150 gr |  |  |  |

| All items | Delivery of all items to following address: Pančevo City Firefighters’ Association, 96 Žarka Zrenjanina Street, 26000 Pančevo, Republic of Serbia. Delivery period: 100 (hundred) days from the commencement date until the provisional acceptance.. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Technical documentation for equipment (Operating manuals/ Users Guide/ Equipment operating instructions/ Cleaning procedures/ Maintenance procedures/ Calibration procedures) upon delivery. For all the required characteristics, the bidder is obliged to submit appropriate catalogues, brochures, declarations, and the like, in Serbian or translated into Serbian, if the offered product comes from a foreign manufacturer, on which the products and their required characteristics will be marked. |  |  |  |
| All items | Warranty Warranty period 365 days from the issuance of PAC (Provisional Acceptance Certificate) in accordance with article 32 SC and GC of the Contract. Offer must include warranty service description including: - Service organisation contact data including name, postal address, telephone number, fax number and e-mail address; - Guaranteed that any requests for services will be attended to within 48 hours; - Guarantee that all items can be repaired or alternatively replaced within a maximum of 7 days; - Letter of confirmation that genuine spare parts and consumables will be available for a period of minimum 3 years from the date of final acceptance of the equipment. |  |  |  |
| All items | Commercial warranty 365 days from the issuance of FAC (Final Acceptance Certificate) in accordance with the conditions laid down in Article 32 of the General Conditions and Article 33 of the Special Conditions. Detailed description of the organisation of the proposed service and description of the Manufacturer’s commercial warranty shall be included in the offer. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >